TENDER TEXT IP1-HMT V.07

| Standards | Car lifting system with driver on board, certified by Inspection Body for the conformity to IX attachment to 2006/42/CE norm. |
| --- | --- |
| General specification | The IP1-HMT V07 model is a CAR LIFT and it was designed to solve parking problems and avoid the ramp to access to the underground garages or to an upper parking level (parking places on the roof of buildings, for example). It occupies about 16 sqm, saving space at all levels. The shaft must be provided at the upper level of an enclosure, in cermet or iron fence high 1200 mm minimum. At the bottom level there must be a pit high 800 mm minimum. The dimension of the shaft, the platform and the technical data are on the attached data sheet. The system is moved by an hydraulic group made by an electric motor and a screw pump that lower and lift the platform dock. The control systems are situated on board the platform, one per each side, working in “hold to run” modus: the driver, remaining inside his car, must push the command and maintain it pushed until the end of the run just sticking out the arm outside the window. The voltage of the control system is 24 V. The user instructions are placed always near to the control system, in a well readable position. |
| Design and description | The system is made by different components: Rails: made by UNP profiles, with 2 fixed columns bolted to the shaft side walls and 2 sliding columns with wheels, on which the platform is fixed. One stiffener beam is fixed under the platform floor, fixed to the sliding columns, in order to distribute the thrust equally on both sides. Parking platform: made by 2 sides in shaped plate, by tubolar frames supporting embossed metal sheets to cover the structure. Lifting system: The parking platform movement is generated by the trust of 2 hydraulic cylinders, one per each long side. On the top of them there is a roller pulley (tackle), the connection between the parking platform and the lifting mechanism is realized through a couple of ropes for each side, each one independent from the other. Directly on the cylinders there are the safety hydraulic locking valves to avoid the runaway descent of the plant in case of pipelines cuttings Steel Structure - FE 360 B quality steel; Mig welding (Gas bare electrode welding) - Non-slip platforms by treadplate iron steel; - Steel protection: hot deep galvanizing steel Oleodinamic plant and hydraulic system - The hydraulic power pack feeds the oleodinamic plant, via the commands on the control panels. It is made by: a tank to contain the oil, a hydraulic gear pump and one or two different motors (380 V, 50 Hz, 5,5 or 7,5 or 9,5 kW power). - The hydraulic power-pack has a pressure gauge to check oil pressure, a locking valve, 3 electro-valves for the descent with a manual emergency command, a flow regulator on descent and a pressure switch to check the weight of the goods on board, an oil level gauge. - N. 2 in tackle single-effect hydraulic cylinders with double rope - Self-leveling hydraulic cylinders by mechanical structural effect - Hydraulic circuit with electrolytic galvanized pipes, fittings and pressure hoses. - The ending parts of the hydraulic pipelines are flexible tubes SAE100R2. - All the components are certified. Electric components: - Electric system wires are in according to CEI 64-2/A Regulations - ON BOARD “ Hold-To-Run” 24 V electric control panel, IP54 protection, manually operated by driver with authorization key switch, operations buttons selector, operation light and emergency push-button (two each unit), for PLATFORM ON BOARD DRIVER OPERATIONS; - FLOOR automatic 24 V electric control panel, IP54 protection, manually operated by caller with authorization key switch, operations buttons selector, operation light and emergency push-button (one each floor), for CALL OF PLATFORM OPERATIONS; - Light barriers/electric eyes barriers on board of platform to ensure correct car stop positioning; - Main switch cabinet with PLC control and self-diagnostic on integrated LCD display IP54 protection with electric and electronic components to operate unit; - Electric motor 380 V., 3ph, 5,5kW (start-up 9 kW), IP54 protection; - All the components are double in the safety circuits with safety modules certified by the suppliers; - Tension control system on ropes; - Auxiliary Power Unit for descent in case of black-out; - On board automatic dialing service (if required) - Automatic opening of the door when the platform arrives at floor; - Interface between doors automation and car lift electric board logic Safety devices which are supplied on IP1 HMT V.08 - Electro-hydraulic one-way solenoid valve functioning during raising and lowering of the unit. - Hydraulic locking valves installed on each cylinder base - Light barriers on board of platform to ensure correct car stop positioning; - Hoisting system with steel ropes and pulleys - ON BOARD Platform lateral polycarbonate steel framed fence protections. |
| Coatings of surfaces | All the surfaces are hot deep galvanized. The cylinders are varnished. In option: TRIPLEX treatment of the surfaces for a durable colour effect. |

ATTACHMENTS:

- Technical data sheet.

- CE certification by IMQ Inspection Body.